FIX IN 5

| Colour | Product Code | Pack Size | Box Qty |
| --- | --- | --- | --- |
| GREY | FIXIN5 | 310ml | 12 |

Product Description

EVERBUILD FIX IN 5, 5 min Fast Fix is an ultra-high strength construction adhesive with an almost

limitless array of applications. The advanced, fibre reinforced, PU formula provides extra strength

and gap filling properties, meaning FIX IN 5 can be used in the most demanding of jobs.

FIX IN 5 bonds virtually all materials including: timber, chip and fibre boards, MDF, coated boards, metals, aluminium, natural stone, concrete, marble, ceramics, plastics, plasterboards, foam (EPS), hard foam, acrylic and much, much more.

Benefits

- Fast, easy application.

- Suitable for interior and exterior use.

- Good chemical resistance.

- Weather resistant.

- Can be sanded, stained, painted or varnished once dry.

- Bridges gaps.

- Sound deadening.

- Excellent primer-less adhesion.

Areas For Use

- Bonding insulation materials and all types of foam

- Repair of loose or torn out screws and hinges

- Fixing wall panels, skirting, cladding etc

- Will bond almost all types of wood

- Installation of cable channels

- .. and much more

Limitations

- At least one of the surfaces to be bonded must be porous.

- Not suitable for polyethylene, polypropylene, Teflon or Nylon.

- Surfaces may be damp, but moisture content of wood must not exceed 30% w/w.

- As product expands slightly on curing, do not use on sheet laminate; use STICK 2 CONTACT ADHESIVE.

- Only apply between +5°C and + 35oC.

- Ensure surfaces are always clamped during use.

Limitations cont.

- Setting times will be increased at low temperatures and low humidity.

- Do not use for children’s toys and articles which can be placed in the mouth by children.

- FOR PROFESSIONAL USE ONLY.

Surface Preparation

The surfaces to be glued should be clean and free from dust, grease and other contaminants. At least one of the materials must be porous. Roughen non-porous surfaces to give a key for the adhesive. To assist cure and achieve maximum bond strength, slightly moisten one surface with a damp cloth. Do not over wet.

Application

Cut tip of cartridge taking care not to damage the thread. Fix nozzle, cut to required aperture and apply using standard sealant gun.

Wearing of gloves is recommended. Apply adhesive evenly to one surface. If necessary, cover large areas with a fine toothed spreader. Assemble parts IMMEDIATELY and clamp for duration of cure (5-10 minutes at 20oc; longer at lower temperatures). After this time, the joint may be easily handled. Full and final bond strength will be achieved after 5 minutes.

NOTE: These products foam slightly on application to fill small gaps. Any excess foam/adhesive must be removed with acetone before the product sets or with a sharp chisel once set.

Specific Data

| Colour | Grey |
| --- | --- |
| Contents | 100% prepolymers and catalyst (solvent free) |
| Viscosity | 420 x103 mPa.s (Brookfield RVT sp7/5rpm) |
| Coverage | One cartridge will cover approx. 1m2 |
| Open time | Approx. 30 seconds at 20°C |
| Cleaner | Everbuild Foam Cleaner or acetone BEFORE curing. After cure, use mechanical abrasion. |

Health & Safety

Consult MSDS for full list of hazards.

Storage

Store in original unopened containers between 5 and 25oC. Reseal immediately once used or disposal will be necessary

Shelf Life

6 months from date of manufacture in original unopened containers.

The technical data contained herein is based on our present knowledge and experience and we cannot be held liable for any errors, inaccuracies, omissions or editorial failings that result from technological changes or research between the date of issue of this document and the date the product is acquired. Before using the product, the user should carry out any necessary tests in order to ensure that the product is suitable for the intended application. Moreover, all users should contact the seller or the manufacturer of the product for additional technical information concerning its use if they think that the information in their possession needs to be clarified in any way, whether for normal use or a specific application of our product. Our guarantee applies within the context of the statutory regulations and provisions in force, current professional standards and in accordance with the stipulations set out in our general sales conditions. The information detailed in the present technical data sheet is given by way of indication and is not exhaustive. The same applies to any information provided verbally by telephone to any prospective or existing customer.